# Method

## Problem Setup

We study multi-task imitation learning where a robot learns a policy from expert demonstrations. At time step t, the agent receives multi-modal observations ot={It(v)}v∈V consisting of multi-view RGB images (e.g., agentview and eye-in-hand). Each task is specified by a natural-language instruction, represented by a pre-computed offline text embedding ℓ∈Rdℓ. The policy outputs continuous robot actions at∈Rda.

Following prior action-sequence policies, we predict a horizon-L action chunk:

at:t+L−1=πθ(ot,ℓ),

where at:t+L−1∈RL×da.

## Diffusion Policy for Action Sequences

Let x0∈RL×da denote the ground-truth action sequence from demonstrations. We model the conditional distribution p(x0∣c) using a diffusion model, where the condition c is derived from visual observations and the offline language embedding.

#### Forward Process.

A forward noising process produces xt from x0 by gradually adding Gaussian noise:

q(xt∣xt−1)=N​αtxt−1,(1−αt)I,

with a predefined noise schedule {αt}t=1T and diffusion horizon T.

#### Training Objective.

We train a noise-prediction model ϵθ(⋅) with the standard ϵ-prediction loss:

L(θ)=Et,x0,ϵϵ−ϵθ(xt,t,c)22,

where ϵ∼N(0,I) and xt is constructed by the closed form: xt=αtx0+1−αtϵ.

#### Inference Hyperparameters.

We explicitly distinguish two hyperparameters: (i) the diffusion horizon T (implemented as num_sampling_steps/n_timesteps), and (ii) the number of reverse sampling iterations K at test time (implemented as ddim_steps for DDIM). This separation is important for fair comparisons and efficiency analysis.

## Transformer–Mamba Heterogeneous Encoder–Decoder Denoiser

We adopt an explicit encoder–decoder denoiser to better utilize multi-modal context. The overall denoiser takes a noisy action sequence xt, diffusion step t, and context c, and predicts ϵ.

#### Multi-modal Context Construction.

A visual encoder maps each RGB view It(v) to a set of tokens. We concatenate multi-view visual tokens with the offline language embedding token(s) to form a context token sequence C. The encoder then computes a context memory:

H=EncTrans(C),

where EncTrans is a Transformer encoder enabling global interactions across modalities.

#### Mamba Decoder for Stable Long-Sequence Denoising.

Given the context memory H and a diffusion time embedding e(t), we decode the noisy action sequence using a Mamba-style sequence model:

ϵ=DecMamba(xt,H,e(t)).

We inject the diffusion-step information via adaptive conditioning (e.g., AdaLN/FiLM-style modulation), which stabilizes denoising across diffusion steps. Intuitively, the Transformer encoder specializes in global cross-modal alignment, while the Mamba decoder provides efficient and stable long-range sequence modeling for action denoising.

## From DDPM to DDIM: Reducing Stochasticity for Long-Horizon Stability

In the baseline setting, DDPM-style sampling injects random noise at each reverse step, which can increase variance in generated action chunks. In long-horizon tasks, such variance may amplify error accumulation and lead to inconsistent task progression. We therefore use DDIM sampling with deterministic setting (η=0), which follows a non-stochastic reverse trajectory while retaining the same learned denoiser. This change improves inference efficiency and reduces sampling variance under the same step budget.

## Closed-Loop Execution via Replanning Frequency replan_every

Action-chunk policies are typically executed in an open-loop manner: the policy predicts L actions and the controller executes the entire chunk without replanning. This can be brittle for contact-rich tasks (e.g., LIBERO-Object) where small state-estimation or actuation errors quickly compound.

We parameterize the execution with a replanning frequency r (implemented as replan_every): the policy replans every r environment steps by generating a new length-L chunk, while only executing the first r actions of the chunk. When r=1, the policy replans at every step (fully closed-loop); when r=L, it reduces to open-loop chunk execution.

Trained action-chunk policy πθ, chunk length L, replanning interval r, initial observation o0, language embedding ℓ t←0 at:t+L−1←πθ(ot,ℓ) Execute action at as the first unused element in at:t+L−1 Observe ot+1 t←t+1

#### Compute–Performance Trade-off.

The overall inference cost scales with both the sampling iterations K and the replanning frequency 1/r. We therefore report success rates together with runtime/throughput metrics, and analyze the performance–compute trade-off across different (K,r) settings.